Conclusions

good

- general understanding of parameters & dependencies has improved considerably since the last meeting

- gun voltage 
design for 140 kV , 7A
with 150 kV voltage capability

- beam parameters
it seems that emittance, energy spread, bunch-length requirements can be met

- buncher section
=0.75 section with =1 continuation seems to be very attractive solution

- problems of how to simulate (and how to define) parasite bunches solved

- HOM dampers are required for buncher section for nominal phase parameters

bad

- what is the maximum magnetic field on injector TW structures we have to plan for (1-2 kG ?)

- gun voltage stabilisation / arc protection problem worse with 140 kV, no progress since last meeting

- satellite bunch, no satisfactory solution yet