The 3 stages of the CTF3

- Preliminary

- Initial

- Nominal

#### CTF3 preliminary stage

- Thermionic gun  (= CLIO gun)

Cathode 0.5 cm2  (2 cm2)

Current 1 - 2 A

Should be adapted to the existing LIL plug-in

- Gun pulser   7 pulses spaced by 420 ns

- LIL Bunching system

PB			B			4 MeV   0.3 A

- LIL Matching system

SNF		QF1	    QD2     QF3

- Beam instrumentation

CTF3 Injector Preliminary stage

Parameters expected at injector exit

New CLIO gun + present LIL bunching system

May 2000

| Parameters | Unit | Preliminary |
| --- | --- | --- |
| Beam energy | MeV | 4 |
| Beam pulse length | s | 2.53 |
| RF pulse length | s |  3.8 |
| Beam current | A | 0.3 - 0.6 |
| Gun current | A | 1 - 2 |
| Useful charge per pulse | nC | 10 |
| Total charge per pulse | nC | 14 |
| Useful bunches per pulse |  | 100 |
| Total bunches/pulse |  | 140 |
| Charge per bunch | nC | 0.1 - 0.2 |
| Bunch spacing | m | 0.1 |
| Bunch length (FWHM) | ps | 14 |
| Bunch length (rms) | mm | 1.8 |
| ### Normalized emittance (rms) | mm-mrad | 50 |
| ### Energy spread ### (single bunch rms) | MeV | < 0. 5 |
| ### Charge variation bunch to bunch | % | < 20 |
| ### Beam repetition rate | Hz | 50 |
| RF repetition rate | Hz | 100 |
| RF fundamental frequency | GHz | 2.99855 |
| Number of klystron 35 MW |  | 1 |
| RF peak power for injector | MW | 30 |

Parameters for the CLIO gun used for the CTF3 Preliminary stage

| Parameters | Unit | Preliminary |
| --- | --- | --- |
| Voltage (for beam) | kV | 90 |
| Voltage (for conditioning) | kV | 100 |
| Length of pulse train | s | 2.6 |
| Gun current | A | 1 - 2 |
| Rise/Fall time | ns |  2 |
| Pulse spacing jitter (420 ns) | ns |   0.5 |
| Voltage stability V/V | % | 0.1 |
| Emittance (norm., edges) | mm.mrad |  15 |
| Beam repetition rate | Hz | 50 |

CTF3 Injector Initial and Nominal stages

Target parameters at injector exit

May 2000

| Parameters | Unit | Initial | Nominal |
| --- | --- | --- | --- |
| Beam energy | MeV |  26 |  26 |
| Beam pulse length | s | 1.54 | 1.54 |
| RF pulse length | s |  1.6 |  1.6 |
| Beam current | A | 3.5 | 3.5 |
| Gun current | A | 7 | 7 |
| Useful charge per pulse | nC | 4893 | 4893 |
| Total charge per pulse | nC | 5382 | 5382 |
| Useful bunches per pulse |  | 4200 | 2100 |
| Total bunches/pulse |  | 4620 | 2310 |
| Charge per bunch | nC | 1.17 | 2.33 |
| Allowed charge in satellite | % |  | <5 |
| Bunch spacing | m | 0.1 | 0.2 |
| Bunch length (FWHM) | ps | < 12 | < 12 |
| Bunch length (rms) | mm | < 1.5 | < 1.5 |
| ### Normalized emittance (rms) | mm-mrad | < 100 | < 100 |
| ### Energy spread ### (single bunch rms) | MeV | < 0.5 | < 0.5 |
| Energy spread (total on flat top incl. Beam loading) | MeV | < 1 | < 1 |
| ### Charge variation bunch to bunch ### (High Frequency: f >> 1/ f ) | % | < 2 | < 2 |
| ### Charge flatness total on flat top ### (Low Frequency: f  1/ f) | % | < 0.1 | < 0.1 |
| ### Beam repetition rate | Hz | 5 | 5 |
| RF repetition rate | Hz | 30 | 30 |
| RF fundamental frequency | GHz | 2.99855 | 2.99855 |
| Number of klystrons 45 MW |  | 2 | 2 |
| RF peak power (MDK decoupled) | MW | 124 (*) | 124 (*) |
| RF peak power (MDK coupled) | MW | 168 (**) | 168 (**) |
| Accelerator structure length | m | 1 | 1 |

(*)  1 x 40 MW (without LIPS on Buncher) + 2.1 x 40 MW  (with LIPS on the 2 ACS)

(**)  2 x 40 MW x 2.1 = 168 MW  (with LIPS on the Buncher and on 2 ACS)

Parameters for the CTF3 gun used for the Initial and Nominal stages

| Parameters | Unit | Initial and Nominal |
| --- | --- | --- |
| Voltage | kV | 150 (*) |
| Pulse length | s | 1.6 |
| Gun current | A | 7 |
| Charge per pulse | C | 11 |
| Rise/ Fall time | ns |  10 |
| Voltage stability V/V | % |  0.1 |
| Charge flatness on flat top | % |  0.1 |
| Emittance (norm., edges) | mm.mrad |  10 |
| Repetition rate | Hz | 5 |

(*) Preliminary value which may be reduced in the course of the injector design study between 90 kV and 150 kV.

Parameters for CLIC / CTF3 Laser

October 1999

| Linear accelerator | CTF-3 | CLIC 3 TeV |
| --- | --- | --- |
| Repetition rate | 5 Hz | 100 Hz |
| Macro-pulse duration / Linac | 1.4 s | 91.6 s |
| Micro-pulse number / Linac | 2145 | 42944 |
| Distance between pulses | 20 cm (667 ps) | 64 cm (2.13 ns) |
| Total train number / Linac | 11 | 704 |
| Pulse number / Train | 195 | 122 |
| Pulse width (FWHH) | 10 ps | 24 ps |
| Charge / pulse | 2.4 nC | 16 nC |
| Total charge / Linac | 5.2 C | 690 C |
| # Charge stability | < 0.5 % rms | 0.1 % rms |
| Photo cathode | Cs2Te | Cs2Te |
| Photo-cathode quantum efficiency @   270 nm | 1.5 % | 1.5 % |
| Laser energy stability (UV) | 0.5 % rms | < 0.5 % rms |
| Laser / RF pulse synchronisation | < 1 ps | < 1 ps |
| Output wavelength | < 270 nm | < 270 nm |
| UV Laser energy / pulse | 0.76 J | 5 J |
| Total Laser energy on the PC | 1.6 mJ | 215 mJ |
| Total efficiency from IR to UV at the photo cathode | 5 % | 5 % |
| IR Laser energy / Linac | 32 mJ | 4.3 J |
| Linac number | 1 | 2 |
| Total IR laser energy | 32 mJ | 8.6 J |
| Laser Average Power | 0.15 W | 860 W |

Planning indicative for the CTF3 Injector

Year 2000

April 2000	Send CLIO gun specifications to LAL

(cahier des charges)

May 2000: 	Decide voltage for CTF3 gun

Send new CTF3 gun specifications to LAL

(cahier des charges)

December 2000:

Complete beam dynamics design studies of the injector (26 MeV) for both the Initial and Nominal stages.

Complete beam loading analysis.

Produce Superfish files defining basic geometry of the Pre-Buncher (PB) and the Buncher (B).

Send PB specification for the Initial stage to LAL

(cahier des charges).

Year 2001

January 2001:	Detail RF design and fabrication drawings for Buncher (CERN).

Start construction of solenoids.

Design and specification of MDK 1.5 GHz for SHB.

Start modifications of LIL front-end for CTF3

Preliminary phase

April 2001:	End of LAL tests of CLIO gun.

May 2001: 	CLIO gun available at CERN

June 2001:	Install CLIO gun in LIL

Start commissioning of injector

Start solenoid measurements

Year 2002

January 2002:	CTF3 gun available  at CERN

Start design of SHB

Construction of MDK 1.5 GHz

Test and commissioning of injector

Year 2003

Construction of SHB's and tests at CERN

Year 2004

January 2004: 	CTF3 Nominal stage

Installation

More details later on…..